CPE301 – SPRING 2019

# Design Assignment 2B

Student Name: John Galanza

Student #: 5002981771

Student Email: galanj1@unlv.nevada.edu

Primary Github address: https://github.com/JohnGalanza

Directory: https://github.com/JohnGalanza/supersmashjoe

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmega328p

- Multifunctional Shield

- USB cable

- Headphones

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

Task 1 (Initial code used from previous DA)

; DA2T2.asm

;

CBI DDRC,2   	 ;makes PC2 an input

LDI R19, 0x04

OUT DDRB, R19    ;makes PB2 an output port

L1:    SBIC PINC,2   	 ;Skip if bit PB2 is LOW

RJMP L1   		 ;checks if bit is 0

SBI PORTB,2   	 ;turns on port b

CALL Delay   	 ;delays for 1.250s

CBI PORTB,2   	 ;clears port b

RJMP HERE

Delay:

LDI R16,102

LDI R17,118

LDI R18,194

L2: DEC R18

BRNE L2

DEC R17

BRNE L2

DEC R16

BRNE L2

HERE:RJMP HERE

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Task 1 in assembly (Modified for interrupts using INT0 and PD2)

; DA2B_A.asm

.include<m328pdef.inc>

.ORG 0

JMP MAIN

.ORG 0x02

JMP EX0_ISR

MAIN:

SBI DDRB,2   			 ;Port B2 an output

SBI PORTB,2   			 ;Turn led off

SBI PORTD,2   			 ;activate pull up

LDI R20, 0x2   		 ;Int0 falling edge

STS EICRA,R20

LDI R20, HIGH(RAMEND)    ;setup stack

OUT SPH, R20

LDI R20, LOW(RAMEND)

OUT SPL, R20

LDI R20, 1<<INT0

OUT EIMSK, R20

SEI   					 ;checks INT0 if changed

RJMP MAIN   			 ;Repeats

EX0_ISR:

CBI PORTB, 2   		 ;turn led on

LDI R16,102   			 ;Start of delay for 1.25s

LDI R17,118

LDI R18,194

L2: DEC R18

BRNE L2

DEC R17

BRNE L2

DEC R16

BRNE L2

RETI

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

This shows PD2 active

- SCREENSHOT OF EACH DEMO (BOARD SETUP

- VIDEO LINKS OF EACH DEMO

https://youtu.be/XxjnjKpVzEc

- GITHUB LINK OF THIS DA

https://github.com/JohnGalanza/supersmashjoe/tree/master/DA2B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

John GalanzaT