CPE301 – SPRING 2019

# Design Assignment 5

Student Name: John Galanza

Student #: 5002981771

Student Email: galanj1@unlv.nevada.edu

Primary Github address: https://github.com/JohnGalanza

Directory: https://github.com/JohnGalanza/supersmashjoe/tree/master/DA5

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATmega328p

- usb cord

- LM35

- NRF24L01+ RF Module

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

//DA5 use earlier ADC code to transmit ADC value of internal temp sensor

//between two RF modules

//Atmega will also alternate between TX and RX modes every 0.5s

//Additional commented sections were for testing

#ifndef F_CPU   			 // Sets clock frequency.

#define F_CPU 16000000UL

#endif

#include <avr/io.h>   		 // Includes needed libraries.

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdlib.h>

#include <stdio.h>

#include <string.h>

#include "inc\nrf24l01.c"    //    Include nRF24L01+ library.

#include "inc\nrf24l01-mnemonics.h"

#include "inc\spi.c"

#ifndef BAUD   			 // Sets up UART for printf();

#define BAUD 9600

#endif

#include "inc\STDIO_UART.c"

//also additional code

//#define BAUD_RATE 9600

void print_config(void);    // Functions

void adc_init(void);

//additional testing code below

void USART_init();

void USART_tx_string(char * data);

void USART_send(unsigned char ch);

void USART_print(char* str);

volatile unsigned int adc_temp;

//ends here

volatile bool message_received = false;    // Used in IRQ ISR.

volatile bool status = false;   		 // Used in IRQ ISR.

int tempf = 0;   						 // Used for temperature data.

int main(void)

{

adc_init();   						 // Initializes the ADC.

//USART_init();

char tx_message[32];   			 // Defines string array.

strcpy(tx_message,"Wiggidy Wack");    // Copies string into array.

uart_init();   					 // Initializes UART.

nrf24_init();   					 // Initializes nRF24L01+ and print configuration info.

print_config();   					 // Configures prints.

nrf24_start_listening();   		 // Start listening to incoming messages.

nrf24_send_message(tx_message);   	 // Sends message.

//USART_print("ON");

while (1)

{

ADCSRA |= (1<<ADSC);   				 // Starts conversion.

while((ADCSRA&(1<<ADIF))==0);   		 // Waits for conversion to finish.

ADCSRA |= (1<<ADIF);   				 // Resets flag for conversion.

tempf = ADCL;   						 // Records temp sensor data.

tempf = tempf | (ADCH<<8);

tempf = (tempf/1024.0) * 5000/10;

char temp[5];   						 // Variable used to store tempf string.

itoa(tempf, temp, 10);   				 // Converts tempf integer to string.

//additional code here

//USART_print(temp);

//Above should cause terminal window to pop up

//and also show a value maybe ???

_delay_ms(2000);

message_received = true;

if (message_received)

{

message_received = false;   		 //    Message received, print it.

printf("Message received: %s\n",nrf24_read_message());

_delay_ms(500);

status = nrf24_send_message(temp);    // Send message as response.

if (status == true) printf("Successfully sent message\n");

}

}

}

ISR(INT0_vect)    // Interrupt on IRQ pin.

{

message_received = true;

}

void read_adc(void)

{

unsigned char i = 4;

adc_temp = 0;

while(i--)

{

ADCSRA|=(1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp += ADC;

_delay_ms(50);

}

adc_temp = adc_temp/4;   		 //averages a few samples

}

void USART_init(void)

{

UBRR0L = 8;

UCSR0C = (1<<UCSZ01)|(1<<UCSZ00);    //asynchronous 8 N 1

UCSR0B = (1<<TXEN0)|(1<<RXEN0);   	 //enable receiver, transmitter & RX interrupt

}

void USART_send(unsigned char ch)

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = ch;

}

void USART_print(char* str)

{

int i = 0;

while (str[i] != 0)

{

USART_send(str[i]);   		 //increments i to go through the whole string

i++;

}

}

//Sends data to serial port

void USART_tx_string(char *data)

{

while((*data!= '\0'))

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = *data;

data++;

}

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG    0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA   		 0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR   	 0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR   	 0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH   		 0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP   	 0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS   	 0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE   	 0x%x\n",data);

printf("-------------------------------------------\n\n");

}

void adc_init (void)    // Sets up and enables ADC.

{

ADMUX = (0<<REFS1)|   		 // Reference Selection Bits.

(1<<REFS0)|   		 // AVcc - external cap at AREF.

(0<<ADLAR)|   		 // ADC Left Adjust Result.

(0<<MUX2)|   		 // Analog Channel Selection Bits.

(0<<MUX1)|   		 // ADC0 (PC0).

(0<<MUX0);

ADCSRA = (1<<ADEN)|   		 // ADC Enable.

(0<<ADSC)|   		 // ADC Start Conversion.

(0<<ADATE)|   		 // ADC Auto Trigger Enable.

(0<<ADIF)|   		 // ADC Interrupt Flag.

(0<<ADIE)|   		 // ADC Interrupt Enable.

(1<<ADPS2)|   		 // ADC Pre-scaler Select Bits.

(0<<ADPS1)|

(1<<ADPS0);

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Insert only the modified sections here

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/JohnGalanza/supersmashjoe/tree/master/DA5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

John Galanza