CPE301 – SPRING 2019

# MIDTERM 1

Student Name: John Galanza

Student #: 5002981771

Student Email: galanj1@unlv.nevada.edu

Primary Github address:https://github.com/JohnGalanza

Directory: https://github.com/JohnGalanza/supersmashjoe

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/Midterm, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmega 328p

- LM 35 temperature sensor

- FTDI chip

- female to male jumper wires

- breadboard

- laptop

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#define BAUD_RATE 9600

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <stdlib.h>   				 //all the headerss

void read_adc(void);   			 //Function declarations

void adc_init(void);

void USART_init();

void USART_tx_string(char * data);

void USART_send(unsigned char ch);

void USART_print(char* str);

volatile uint8_t OVFCount;

volatile unsigned int adc_temp;

char outs[20];

int main(void)

{

adc_init();   						 //Initializes ADC

USART_init();   			 //Initialize USART

while (1)

{

ADCSRA|=(1<<ADSC);

while((ADCSRA & (1<<ADIF))==0);

ADCSRA |= (1<<ADIF);

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0)* 5000/10;

USART_send((a/100)+'0');

a = a % 100;

USART_send((a/10)+'0');

a = a % 10;

USART_send((a)+'0'); //converts the number

USART_send('\r');

_delay_ms(1000);

char setMUX[] = "AT+CIPMUX=0\r\n"; //Repeating the steps ESPlorer used

USART_print(setMUX); //Sends string to USART using USART_send to send each character of string

_delay_ms(1000);

char thingSpeakINIT[] = "AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n" ;

USART_print(thingSpeakINIT);

_delay_ms(1000); //add in delay to allow for proper interactions

char SENDthingSpeak[] = "AT+CIPSEND=80\r\n"; //saying we'll send more data than we actually will

USART_print(SENDthingSpeak);

_delay_ms(1000);

char GETthingSpeak;  //sends data to thingSpeak

snprintf(GETthingSpeak, sizeof(GETthingSpeak), "GET https://api.thingspeak.com/update?api_key=R9HDZJDAPCP0G215&field1=0", a);

USART_print(GETthingSpeak);

_delay_ms(1000);

char thingSpeakClose[] = "AT+CIPCLOSE\r\n";

USART_print(thingSpeakClose);

_delay_ms(1000);

}

return 0;

}

void adc_init(void)

{

//set up and enable ADC

ADMUX = (0<<REFS1)| //Reference selection bits

(1<<REFS0)|    //AVcc - external cap at AREF

(0<<ADLAR)|    //ADC Left adjust result

(0<<MUX2)|    //analog channel selection bit

(0<<MUX1)|    //chooses pc5 pin28

(0<<MUX0);

ADCSRA =(1<<ADEN)|

(0<<ADSC)|

(0<<ADATE)|

(0<<ADIF)|

(0<<ADIE)|

(1<<ADPS2)|    //prescaler

(0<<ADPS1)|

(1<<ADPS0);

}

void read_adc(void)

{

unsigned char i = 4;

adc_temp = 0;

while(i--)

{

ADCSRA|=(1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp += ADC;

_delay_ms(50);

}

adc_temp = adc_temp/4;   		 //averages a few samples

}

void USART_init(void)

{

UBRR0L = 8;

UCSR0C = (1<<UCSZ01)|(1<<UCSZ00);    //asynchronous 8 N 1

UCSR0B = (1<<TXEN0)|(1<<RXEN0);   	 //enable receiver, transmitter & RX interrupt

}

void USART_send(unsigned char ch)

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = ch;

}

void USART_print(char* str)

{

int i = 0;

while (str[i] != 0)

{

USART_send(stri[i]);   		 //increments i to go through the whole string

i++;

}

}

//Sends data to serial port

void USART_tx_string(char *data)

{

while((*data!= '\0'))

{

while(!(UCSR0A & (1<<UDRE0)));

UDR0 = *data;

data++;

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/CVL937mhxgE

- GITHUB LINK OF THIS DA

https://github.com/JohnGalanza/supersmashjoe/tree/master/Midterm%201

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

John Galanza